Reconnect plugin for Sprint Layout 6.0 version 2022
Sprint60ReconnectPlugin.exe

Written by Ben Bulsink, 03-03-2022 Vs 1.0. 
benbulsink@outlook.com. This tool may be used and distributed freely.

Version 1.0: Initial version, based on Reconnect.exe version 1.4 (may2020)

Purpose

When you are designing a more complex PCB using Sprint-Layout 6.0, and you are using Rubber band or Air wire connections to add all connections before routing (Rats nest), and then you are placing all components to fit optimal on the PCB area, it happens that the made connections are not optimal anymore to be used for the routing. This tool can be used to reshuffle the connections so that the shortest connections are made for the given component placement.

Placement is an iterative process: The rubber bands show where long traces are needed. When a component position is changed, the reconnecting may find shorter distances to the same net. Shift and reconnect frequently

How to use this tool

The plugin is based on the information given in the Sprint Layout help information > Miscellaneous Functions > Plugins

Copy the Sprint60ReconnectPlugin.exe file to a favourite map.

In Sprint Layout, select this file as plugin:

Now, when your design, containing rubber bands connections, is opened in Sprint Layout, take care to have no items selected (or select all). Then, following the above menu picture, choose Run Plugin…

The rubber band connections will be optimized, and Sprint Layout will show the new configuration. Notice that Sprint Layout will show all items selected, after running the plugin.

The plugin can raise error codes:

Error 129: No command line arguments found

Error 130: Only incomplete board transferred. Unselect all or Select all

Error 131: Error opening file

Error 132: Too many connected pads (max 999)

Always, a technical report file  “%AppData%\Local\Temp\Reconnect_rep.txt” is generated.

Examples (left side before, right side after optimising the connections)

Inefficient connections between R1, R2 and R3 are optimized for the given component placement.

The shortest connection is made in this extreme example

with two nets

Below right: Loops in the net are removed